Приложение

## Список комманд TNC

Далее представлен список комманд  встроенного TNC. Между самой коммандой и параметром должен быть пробел. Также пробел должен быть между двумя параметрами.

| Комманда | Короткая форма | По умолчанию | Параметры | ## Описание |
| --- | --- | --- | --- | --- |
| AUTOLF | AU | ON | ON/OFF | When ON, sends a line feed (LF) to the PC after each carriage return (CR). |
| BEACON | B | EVERY 0 | EVERY/ AFTER n ( n=0 – 250 ) | If set to EVERY, sends a beacon packet at intervals of the specified period (n). If set to AFTER, sends a beacon packet only once after the specified period (n). The unit of n is 10 seconds. |
| BTEXT | BT | - | 0 –159 characters | Specifies the content of the data portion of a beacon packet. |
| CALIBRAT | CAL | - | - | Sends a space/mark square wave (50/50 ratio). Enter Q to exit Calibrate mode and restore the Command mode. |
| CHCAK | CH | 30 | 0 – 250 | Specifies the interval from signal drop-out until execution of disconnection. The unit of the parameter is 10 seconds. |
| CONNECT | C | - | Call1 (VIA call1, call3, ....call9) | Sends a connect request. Call 1 is the call sign of the station to be connected to. Calls 2 to call 9 are call signs of stations to be digipeated through. |
| CONVERSE | CONV or K | - | - | Causes the TNC to enter Converse mode. Press [Ctrl]+[C] to restore the Command mode. |
| CPACTIME | CP | OFF | ON/OFF | When ON and in Converse mode, sends a packet at intervals of the period determine by PACTIME |
| CR | CR | ON | ON/OFF | When ON, appends a carriage return (CR) to all packets to be sent. |
| DISCONNE | D | - | - | Sends a disconnect request. |
| DISPLAY | DISP | - | - | Causes the TNC to display the current status of all the commands. You can also specify a class identifier A, C, H, I, L, M, or T to display the status of only the desired command class. Enter a space between the command name and a class identifier; ex. DISPLAY H. A (ASYNC): RS-232C port parameters. C (CHAR) : Special TNC characters H (HEALTH): Counter parameters. I (ID): ID parameters. L (LINK): TNC-to -TNC link status. M (MONITOR): Monitor parameters. T (TIMING): Timing parameters. |
| DWAIT | DW | 30 | 30 | Specifies the interval from no carrier detection until execution of transmission. The unit of the parameter is 10 milliseconds. |
| ECHO | E | ON | ON/OFF | When ON, causes the TNC to echo received characters to the computer. |

42